- Architecture layout

- IMU + magnetometer

  - IMU acts as a sensor hub(9KB FIFO) to store the data from the magnetometer

    - Reason: save power / unified timestamp / release the loading of IMU

  - How to connect

    - Sensor hub / AUX I2C

    - SCL / SDA of the magnetometer connect to AUX I2C of IMU

  - How to boot

    - Init stage(ESP behavior)

      - ESP init I2C (ESP & IMU)

      - Set IMU (ODR / FIFO / interrupt)

      - Set Sensor Hub interaction

        - Which external sensor (e.g., magnetometer)

        - Which control register

        - Which data register

        - Set frequency (same as acc / gyro)

    - Runtime

      - IMU

        - periodically read the external sensor(e.g., megnetometer), and put it into the 9KB FIFO

      - ESP

        - Wait for watermark interrupt or interrupt to read 9KB FIFO

        - Get the accel/gyro + mag + timestamp

  - Github: https://github.com/STMicroelectronics/STMems_Standard_C_drivers/tree/master/lsm6dso_STdC/examples

  - Note:

    - Even though the program is only flashed into the ESP, the IMU can still read the data from the magnetometer actively.

    - ESP has to set the Sensor Hub in the IMU

    - IMU would get the data from the magnetometer, and move it to the FIFO based on the instruction given by the ESP

  - SparkFun Provided API

    - readRegister()

    - writeRegister()

    - readMultipleRegisters()

  - set the LSM6DSO as the sensor hub to control LIS2MDL, and store the data in FIFO

    - set sensor hub registers

    - set FIFO

    - activate master

    - read FIFO tag